Outlines:

- Introduction

- Types of Electrical Machines

- Types of 3-Ph induction motor

- Basic Principle

- Working Principle

- Compnents of induction motor

- Application

- Comparision between 1-ph I.M. & 3-ph I.M.

	Electrical Machines are Electromechanical energy converson devices.

	Electrical motor coverts electrical energy into mechanical energy and electrcal generators are vice versa.

	Most of the Industrial load are inductive, especially 3-ph induction motors are widely used in industrial applications for continious operation.

# 1.AC Machines 2.DC Machines

AC Machines:-

- 1-Ph Induction Motor

- 3-Ph Induction Motor

- Synchronous motor

- 1- Ph Transformer

- 3-ph Transformer

- Altrnator or AC Generator

DC Machines:-

-DC Motor

-DC Generator

DC Motor:

- DC Shunt Motor

- DC Series Motor

- DC Compound Wound Motor

DC Generator

- DC Shunt Generator

- DC Series Generator

- DC Compound Wound Generator

Principle of Operation

 A Rotating Magnetic field (RMF) is set up in the stator when a 3- Phase supply is given.

 The stationary rotor cut the revolving field and  due to electromagnetic induction an e.m.f. is induced in the rotor conductor.

 As the rotor conductor is short circuited current flows through them.

 It becomes a current carrying conductor in magnetic field and start rotating.

Slip of an Induction Motor

  - Induction	motor	rotor	always	rotate	at	a	speed	less	than

synchronous speed.

  - The difference between the flux (Ns) and the rotor speed (N) is called slip.

  - % Slip (s) =	Ns – N

Ns

*100

Where Ns=Synchronous Speed N= Actual Speed of rotor

  - Slip speed = Ns – N

- Frame

- Stator

- Stator Winding

- Rotor

- Rotor Winding

- Cooling Fan

- Bearings Frame:

Frame provides mechanical support to the stator and rotor. It is made from casting materials.

Stator:

It is stationary part of induction motor. It consists stator winding

It is housed on the motor frame.

Arrangement of stator in 3-Ph induction motor are given below:-

Image of Rotor & Stator Winding of 3 phase Induction Motor

Components of 3-ph Induction Motor

- Rotor:-

- It is the rotating part of the induction motor.

- It is housed on the shaft of the induction motor.

- It has two ends, one is called Driving end and another is called non-

Driving end.

- Mechanical load is connected on driving end while cooling fan is connected on non-driving end.

- Both the ends are connected with bearings for free rotation means of reduced friction losses.

- Arrangement of rotor of induction motor are given below:

Stator Windings:

	Star  connected

	Delta connected

Rotor Windings:

	It is wound as rotor bars and short circuited at both the ends through end rings.

According to rotor construction it can be Classified in two category according to rotor Construction:-

- Squirrel cage induction motor

- Slip ring induction motor

Squirrel cage Induction Motor:-

	Most of the application of industrial as well as domestic are this type of induction motor.

	Its construction is simple and rugged.

	Cheap copare to slipring IM

	Maintainance is easy

	Cost is less compare to slipring IM

	Can use at explosive area

	Starting torque is low compare to slip ring IM

	Application:-

	Leath machine, Compressors,centrifugal pump, in agriculture etc. where cost is most important factor

Slipring Induction Motor:-

- It has high starting torque compare to squirrel cage IM

- Construction is complicated

- Maintanance cost is high compare to squirrel cage IM

- Starter requires compulsory

- Can not use at explosive area

- Application:-

- Crane,hoist,lift and wherehigh starting torque is required