MILING OPERATION

EXAMPLE 5.4   for the work piece shown in the figure below, perform the end milling operation to smoothen its edges (remove burns from the edges). Use end mill cutter of 25mm diameter. Take feed rate of 30mm per minute. Assume target point to be located at x = -30mm, note that holes are used for locating and positioning the work piece.

NC Part  program     N00   G21  G90 X-30.0 Y-30.0 Z10.0

N05   G00 Z-20.0 S800 M03

N10   G01 G94   G42 X175.0 Y0 D4 F30

N15   G01 X175.00   Y0.0

N20   G17 G03 X200.0 Y25.0 R25.0

N35   G01 X25.0 Y100.0

N40   G17 G17   G03   XO   Y75.0    R25.0

N45   G01 X0.0   Y25.0

N50   G17   G03   X25.0   X25.0   Y0.0   R25.0

N55   G00   G40   X0Y-30

N60   M05   M30

Explanation                Input   values in mm, absolute dimensioning, defining axis

Origin.

Rapid traverse of tool to cutter depth, spindle start rotating in

Clockwise  direction.

Cutter reach the starting position, cutter diameter saved in

Register no.4.

Start milling lower edge.

XY plane selection, circular interpolation in anticlockwise direction.

End mill right edge.

XY plane selection, circular interpolation in anticlockwise direction.

End mill upper edge.

Circular   interpolation in X-Y plane in anticlockwise

direction.

End mill left edge

Circular   interpolation in X-Y plane in anticlockwise

direction.

Rapid exit from part, cancel offset compensation.

Spindle stop, end of program, machine stop and tape rewind.

MILLING OPERATION

EXAMPLE 5.5 	Write a NC part program to drill four holes in a work piece using a drilling machine as shown in figure below. The work piece is a rectangular place of 200 mm x 100mm 10mm. the diameter of holes is 10mm per revelation and spindle speed of 800 revolutions per minute. Assume the target point is located at x=-10mm. the wok piece  surface is  fixtured  such  that its bottom  surface is  30 mm  above the  machine tool  table  surface  so as  to provide sufficient  clearance beneath the part for  drilling  through holes.

NC Part Program

N01  G21 G90 G92 X -10 Y-10 Z10 EOB

N02  G00 X20.0  Y10.0 EOB

NO3 G01 G95 Z- 15.O -15.0 F0.10 800   M03 EOB

N04   G01 Z10.0   EOB

N05   G00   X180.0 Y10.0 EOB

N06   G01   G95   Z   - 15.0   F0.10   EOB

N07   G01   Z10.0   EOB

N08   G00   X80.0   Y90.0   EOB

N09   G01   G95   Z- 15.0   F0.10   EOB

N00   G01   Z10.0   EOB

N11   G00. X20.0. Y90.0 EOB

N12   G01.G95 Z – 15.0 F0.10  EOB

N13   G00 X- 10 Y-10 EOB

N14   G00  X- 10Y-10 EOB

N14   M05   M30   EOB

Explanation            	Input values in mm, absolute dimensioning, defining origin

Of axis.

Rapid traverse of tool to first hole  position, i.e. lower left hole

Drilling lower left hole

Taking out drill bit from the hole

Rapid traverse to lower right hole position

Drill the hole

Taking out drill bit from the hole

Rapid traverse to upper right hole position

Drill the hole

Taking out drill bit from the hole

Rapid traverse to upper left hole position

Drill the hole

Taking out drill bit from the hole

Rapid traverse to target point

Spindle stop, end of program, machine stop and tape rewind.

.