Measurement: involves using an instrument as a physical means

of determining a quantity / variable.

Or

is act of assigning a number to an attribute or physical quantity.

It must contain some unit according to some set of rules.

Or

is act, or the result, of a quantitative comparison between a

given quantity and a quantity of the same kind chosen as unit.

❖ Instruments: a device for determining the value / magnitude of a

quantity/ variable.

❖ Instrumentation: measurement of non-electrical quantities by

means of electrical methods connected / integrated alone with

the signal conditioning circuits.

Why do we need measurement?

• To monitor (parameters), control and experimental engineering

analysis.

❖ Scale: Nominal scale (classification/ rough idea); ordinal scale

(ranking); interval scale (range), and ratio scale (0 has no

meaning in this scale).

• The basic moving coil system

generally referred to as a d’Arsonval

meter movement or Permanent

Magnet Coil (PMMC) meter

movement.

• Current-sensitive device capable of

directly measuring only very small

currents.

• Range: 10μA -100mA

• Coil resistance: 10Ω – 1kΩ

• Usage:

✓ dc PMMC ammeters and voltmeters

✓ ac PMMC ammeters and voltmeters

(with rectifiers).

Current from the circuit in which measurements are being made with the meter passes

through the windings of the moving coil. Current through the coil causes it to behave as

an electromagnet with its own north and south poles. The poles of the electromagnet

interact with the poles of the permanent magnet, causing the coil to rotate. The pointer

deflects up scale whenever current flows in the proper direction in the coil. For this

reason, all dc meter movements show polarity markings. The principle on which

a PMMC is working is

that a torque is excited

on a current-carrying

coil placed in the field

of a permanent

magnet. Deflecting Torque Equation of PMMC Instrument

Let, B = flux density in the air gap (wb/m3)

i = current in the coil (A) ; l = effective axial length of the coil (m);

b = breadth of the coil (m) ; n = number of turns of the coil.

✓Now force on one side of the coil is

F = Biln (N)

✓Torque on each side of the coil,

T = F x b/2 (N)

Whereas, b/2 = distance from axis of rotation Benefits

▪ sensitive to the small current

▪ accurate and reliable

▪ uniform scale up to 270 degree or more

▪ effective built-in damping

▪ low power consumption: 25 μW – 200 μW

▪ free from hysteresis and not affected by the external fields

because of its permanent magnet shields the coil from external

magnetic fields.

▪ easily adopted as a multi-range instrument.

Limitations

▪ This type of instrument suits for DC only. In AC, the instrument does

not operate because of equal amount force experienced by the pointer

but opposite direction. Due to the inertia of the pointer, it (pointer)

remains at zero position.

▪ The moving system is highly delicate so needs careful handling.

▪ the coil being very fine, can not withstand prolonged overloading.

▪ Costlier

▪ Error occur at the later stage due to Ageing effect (permanent magnet

and control spring).

Voltmeter Loading Effects

When a voltmeter is used to measure the voltage across a circuit

component, the voltmeter circuit itself is in parallel with the circuit

component. Since the parallel combination of two resistors is less than

either resistor alone, the resistance seen by the source is less with the

voltmeter connected than without. Therefore, the voltage across the

component is less whenever the voltmeter is connected. The decrease in

voltage may be negligible or it may be appreciable, depending on the

sensitivity of the voltmeter being used. This effect is called voltmeter

loading. The resulting error is called a loading error

Ammeter insertion effects

❖ Inserting an ammeter in a circuit always increases the resistance of the

circuit and reduces the current in the circuit. This error caused by the

meter depends on the relationship between the value of resistance in the

original circuit and the value of resistance in the ammeter.

Moving Iron type Instruments

Electromechanical Indicating Instruments

❖ Construction and basic principle operation

▪ Moving-iron instruments are generally used to measure

alternating voltages and currents.

▪ In moving-iron instruments the movable system consists of one

or more pieces of specially-shaped soft iron, which are so

pivoted as to be acted upon by the magnetic field produced by

the current in coil.

• Range: 10mA -100A

• Usage:

✓dc MI ammeters and voltmeters

✓ac MI ammeters and voltmeters

There are two general types of moving-iron instruments namely:

• Repulsion (or double iron) type (figure 1)

• Attraction (or single-iron) type (figure 2)

The brief description of different components of a moving-iron

instrument is given below:

▪ Moving element: a small piece of soft iron in the form of a vane or

rod.

▪ Coil: to produce the magnetic field due to current flowing through it

and also to magnetize the iron pieces.

▪ In repulsion type, a fixed vane or rod is also used and magnetized

with the same polarity.

▪ Control torque is provided by spring or weight (gravity).

▪ Damping torque is normally pneumatic, the damping device

consisting of an air chamber and a moving vane attached to the

instrument spindle.

▪ Deflecting torque produces a movement on an aluminium pointer

over a graduated scale. How it works?

• The deflecting torque (Td) in any moving-iron instrument is

because of forces on a small piece of magnetically ‘soft’ iron that

is magnetized by a coil carrying the operating current.

• In repulsion type moving–iron instrument consists of two

cylindrical soft iron vanes mounted within a fixed current-carrying

coil. One iron vane is held fixed to the coil frame and other is free to

rotate, carrying with it the pointer shaft. Two irons lie in the

magnetic field produced by the coil that consists of only few turns

if the instrument is an ammeter or of many turns if the

instrument is a voltmeter.

▪ Current in the coil induces both vanes to become magnetized and

repulsion between the similarly magnetized vanes produces a

proportional rotation. The deflecting torque is proportional to

the square of the current in the coil, making the instrument

reading is a true ‘RMS’ quantity. Rotation is opposed by a

hairspring that produces the restoring torque. Only the fixed coil

carries load current, and it is constructed so as to withstand high

transient current.

▪ Limitation: Moving iron instruments having scales that are

nonlinear and somewhat crowded in the lower range of

calibration. Ammeter

▪ Instrument used to measure current in the circuit.

▪ Always connected in series with the circuit and carries the current

to be measured.

▪ This current flowing through the coil produces the desired

deflecting torque.

▪ It should have low resistance as it is to be connected in series.

Voltmeter

• Instrument used to measure voltage between two points in a

circuit.

• Always connected in parallel.

• Current flowing through the operating coil of the meter produces

deflecting torque.

• It should have high resistance. Thus, a high resistance of order of

kilo ohms is connected in series with the coil of the instrument. Benefits

❖ The instruments are suitable for use in AC and DC circuits.

❖ The instruments are robust, owing to the simple construction of

the moving parts.

❖ The stationary parts of the instruments are also simple.

❖ Instrument is low cost compared to moving coil instrument.

❖ Torque/weight ratio is high, thus less frictional error. Limitations

▪ Error due to variation in temperature.

▪ Error due to friction is quite small as torque-weight ratio is high in

moving coil instruments.

▪ Stray fields cause relatively low values of magnetizing force

produced by the coil. Efficient magnetic screening is essential to

reduce this effect.

▪ Error due to variation of frequency causes change of reactance of

the coil and also changes the eddy currents induced in

neighboring metal.

▪ Deflecting torque is not exactly proportional to the square of the

current due to non-linear characteristics of iron material (I2R loss).

Electrodynamometer types of instruments are used as a.c.

voltmeters and ammeters both in the range of power frequencies

and lower part of the audio power frequency range. They are used

as wattmeters, voltmeters and with some modification as power

factor meters and frequency metersOperating Principle

• It would have a torque in one direction during one half of the cycle and an

equal effect in the opposite direction during the other half of the cycle.

• If the frequency were very low, the pointer would swing back and forth

around the zero point.

• However, for an ordinary meter, the inertia is so great that on power

frequencies the pointer does not go very far in either direction but merely

stays (vibrates slightly) around zero.

• If, however, we were to reverse the direction of the flux each time the

current through the movable coil reverses, a unidirectional torque would be

produced for both positive and negative halves of the cycle.

• In electrodynamometer instruments the field can be made to reverse

simultaneously with the current in the movable coil if the field (fixed) coil is

connected in series with the movable coil.

Construction

Fixed Coils

• The field is produced by a fixed coil.

• This coil is divided into two sections to give a more uniform field near the centre and

to allow passage of the instrument shaft.

Moving Coil

• A single element instrument has one moving coil.

• The moving coil is wound either as a self-sustaining coil or else on a non-metallic

former.

• A metallic former cannot be used as eddy current would be induced in it by the

alternating field.

• Light but rigid construction is used for the moving coil.

• It should be noted that both fixed and moving coils are air cored.

Control

• The controlling torque is provided by two control springs.

• These springs act as leads to the moving coil.

Moving System

• The moving coil is mounted on an aluminum spindle.

• The moving system also carries the counter weights and truss type pointer.

• Sometimes a suspension may be used in case a high sensitivity is desired.

Damping

• Air friction damping is employed for these instruments and is provided by a

pair of aluminum vanes, attached to the spindle at the bottom.

• These vanes move in sector shaped chambers.

• Eddy current damping cannot be used in these instruments as the

operating field is very weak (on account of the fact that the coils are air cored)

and any introduction of a permanent magnet required for eddy current

damping would distort the operating magnetic field of the instrument.

Advantages

i) These instruments can be used on both a.c & d.c

ii) Accurate rms value

Disadvantages

(i) They have a low torque/weight ratio and hence have a low sensitivity.

(ii) Low torque/weight ratio gives increased frictional losses.

(iii) They are more expensive than either the PMMC or the moving iron type

instruments.

(iv) These instruments are sensitive to overloads and mechanical impacts.

Therefore, they must be handled with great care.

(v) The operating current of these instruments is large owing to the fact that

they have weak magnetic field. The flux density is about 0.006 Wb/m as

against 0.1 to 0.5 Wb/m in PMCC instruments

(vi) They have a non-uniform scale.